# DEPARTMENT OF INFORMATION TECHNOLOGY

GOVERNMENT COLLEGE OF ENGINEERING,

AMRAVATI

B.TECH-THIRD SEMESTER

LAB RECORD FOR

ITU325

DIGITAL LOGIC DESIGN LABORATORY

Submitted by

Name of Student	     :     Anushka Ganesh Narkhede

Registration ID       :     20007020

Submitted By          :     Dilip R. Uike

Submitted to:

GOVERNMENT COLLEGE OF ENGINEERING,

AMRAVATI

GOVERNMENT COLLEGE OF ENGINEERING, AMRAVATI

(An Autonomous Institute of Govt. of Maharashtra)

DEPARTMENT OF INFORMATION TECHNOLOGY

WINTER-2021

CERTIFICATE

This is to certify that this lab record contains the bonafide lab work of ANUSHKA GANESH NARKHEDE having   ID 20007020 of semester III of B.Tech in Information Technology during academic year 2021-22 for the  Course Title:- Digital Logic Design Lab     Course Code:-ITU325

Date:-

Head of Dept.						Faculty

(Prof.A.W.Bhade) 					    (Prof. Dilip Uike)

Course Objectives :

At the end students should

- To introduce the concepts and techniques associated with the number systems and codes.

- To minimize the logical expressions using Boolean postulates.

- To design various combinational and sequential circuits.

- To provide with an appreciation of applications for the techniques and mathematics used in this course.

Table  Of  Contents

| Experiment No. | Date | Title Of Experiment | Page No. | Remark |
| --- | --- | --- | --- | --- |
| 1 |  | Introduction to VHDL. |  |  |
| 2 |  | Simulation of basic gates AND,OR,NOT in vhdl using dataflow modeling. |  |  |
| 3 |  | Simulation and implementation of gate (NAND, NOR, EXOR) in VHDL. |  |  |
| 4 |  | Simulation & Implementation of 8:1 Multiplexer |  |  |
| 5 |  | Design Half Adder and full adder circuit using VHDL. |  |  |
| 6 |  | Design & implement binary to gray code conversion in vhdl. |  |  |
| 7 |  | To simulate & implement 3:8 decoder using VHDL. |  |  |
| 8 |  | To design & implement the D- Flip Flop using VHDL. |  |  |
| 9 |  | To design and implement 3 bit synchronous up/down counter using VHDL. |  |  |

PRACTICAL NO – 1

Here we should specify the entity name for which we are writing the architecture body. The architecture statements should be inside the begin and end keyword. Architecture declarative part may contain variables, constants, or component declaration.

Configuration:

If an entity contains many architectures and  any one of the possible architecture binding with its entity is done using configuration. It is used to bind the architecture body to its entity and a component with an entity.

Syntax:

configuration configuration_name of entity_name is

block_configuration;

end  configuration_name.

Block_configuration defines the binding of components in a block. This can be written as

for block_name

component_binding;

endfor;

block_name is the name of the architecture body. Component binding binds the components of the block to entities. This can be written as,

for component_labels:component_name

block_configuration;

endfor;

Package declaration:

Package declaration is used to declare components, types, constants, functions and so on.

Syntax:

package package_name is
       Declarations;
    end package_name;

Package body:

A package body is used to declare the definitions and procedures that are declared in corresponding package. Values can be assigned to constants declared in package in package body.

Syntax:

packagebody package_name is
        Function_procedure definitions;
    end package_name;

The internal working of an entity can be defined using different modeling styles inside architcture body. They are

1.      Dataflow modeling.

2.      Behavioral modeling.

3.      Structural modeling.

Structure of an entity:

Let’s try to understand with the help of one example.

Dataflow modeling:

In this style of modeling, the internal working of an entity can be implemented using concurrent signal assignment.

Let’s take half adder example which is having one XOR gate and a AND gate.

Library IEEE; use IEEE.STD_LOGIC_1164.all; entity ha_en is port(A,B:inbit;S,C:outbit); end ha_en; architecture ha_ar of ha_en is begin S<=A xor B; C<=A and B; end ha_ar;

Here STD_LOGIC_1164 is an IEEE standard which defines a nine-value logic type, called STD_ULOGIC. use is a keyword, which imports all the declarations from this package. The architecture body consists of concurrent signal assignments, which describes the functionality of the design. Whenever there is a change in RHS, the expression is evaluated and the value is assigned to LHS.

Behavioral modeling:

In this style of modeling, the internal working of an entity can be implemented using set of statements.

It contains:

- Process statements

- Sequential statements

- Signal assignment statements

- Wait statements

Process statement is the primary mechanism used to model the behavior of an entity. It contains sequential statements, variable assignment (:=) statements or signal assignment (<=) statements etc. It may or may not contain sensitivity list. If there is an event occurs on any of the signals in the sensitivity list, the statements within the process is executed.

Inside the process the execution of statements will be sequential and if one entity is having two processes the execution of these processes will be concurrent. At the end it waits for another event to occur.

library IEEE; use IEEE.STD_LOGIC_1164.all; entity ha_beha_en is port( A :inBIT; B :inBIT; S :outBIT; C :outBIT ); end ha_beha_en; architecture ha_beha_ar of ha_beha_en is begin process_beh:process(A,B) begin S<= A xor B; C<=A and B; endprocess process_beh; end ha_beha_ar;

Here whenever there is a change in the value of a or b the process statements are executed.

Structural modeling:

The implementation of an entity is done through set of interconnected components.

It contains:

- Signal declaration.

- Component instances

- Port maps.

- Wait statements.

Component declaration:

Syntax:

component component_name [is]

List_of_interface ports;

endcomponent component_name;

Before instantiating the component it should be declared using component declaration as shown above. Component declaration declares the name of the entity and interface of a component.

Let’s try to understand this by taking the example of full adder using 2 half adder and 1 OR gate.

library IEEE; use IEEE.STD_LOGIC_1164.all; entity fa_en is port(A,B,Cin:inbit; SUM, CARRY:outbit); end fa_en; architecture fa_ar of fa_en is component ha_en port(A,B:inbit;S,C:outbit); endcomponent; signal C1,C2,S1:bit; begin HA1:ha_en portmap(A,B,S1,C1); HA2:ha_en portmap(S1,Cin,SUM,C2); CARRY<= C1 or C2; end fa_ar;

The program we have written for half adder in dataflow modeling is instantiated as shown above. ha_en is the name of the entity in dataflow modeling. C1, C2, S1 are the signals used for internal connections of the component which are declared using the keyword signal. Port map is used to connect different components as well as connect components to ports of the entity.

Component instantiation is done as follows.

Component_label: component_name port map (signal_list);

Signal_list is the architecture signals which we are connecting to component ports. This can be done in different ways. What we declared above is positional binding. One  more type is the named binding. The above can be written as,

HA1:ha_en port map(A => A,B => B, S => S1 ,C => C1 );

HA2:ha_en port map(A => S1,B => Cin, S=> SUM, C => C2);

Test bench:

The correctness of the above program can be checked by writing the test bench.The test bench is used for generating stimulus for the entity under test. Let’s write a simple test bench for full adder.

library IEEE; use IEEE.STD_LOGIC_1164.all; entity tb_en is end tb_en; architecture tb_ar of tb_en is signal a_i,b_i,c_i,sum_i,carry_i:bit; begin eut:entity work.fa_en(fa_ar) portmap(A=>a_i,B=>b_i,Cin=>c_i,SUM=>sum_i,CARRY=>carry_i); stimulus:process begin a_i<='1';b_i<='1';c_i<='1'; waitfor 10ns; a_i<='0';b_i<='1';c_i<='1'; waitfor 10ns; a_i<='1';b_i<='0';c_i<='0'; waitfor 10ns; if now=30ns then wait; endif; endprocess stimulus; end tb_ar;

Here now is a predefined function that returns the current simulation time

What we saw upto this is component instantiation by positional and by name. In this test bench example the entity is directly instantiated. The direct entity instantiation syntax is:

Component_label: entity entity_name (architecture_name)  
    port map(signal_list);

Result: In this way we studied VHDL language.

Viva Questions ?

1.  What is an entity? What are the components involved in it?

2.  What is architecture?  What are the types of it?

3.   What is structural modeling?

PRACTICAL NO – 2

- AND Gate

Program:

-- import std_logic from the IEEE library

library IEEE;

use IEEE.std_logic_1164.all;

-- this is the entity

entity ANDGATE is

port (

IN1 : in std_logic;

IN2 : in std_logic;

OUT1: out std_logic);

end ANDGATE;

architecture RTL of ANDGATE is

begin

OUT1 <= IN1 and IN2;

end RTL;

- OR Gate

Program:

-- import std_logic from the IEEE library

library IEEE;

use IEEE.std_logic_1164.all;

-- this is the entity

entity ORGATE is

port (

IN1 : in std_logic;

IN2 : in std_logic;

OUT1: out std_logic);

end ORGATE;

architecture RTL of ORGATE is

begin

OUT1 <= IN1 or IN2;

end RTL;

- NOT Gate

Program:

-- import std_logic from the IEEE library

library IEEE;

use IEEE.std_logic_1164.all;

-- this is the entity

entity NOTGATE is

port (

IN1 : in std_logic;

OUT1: out std_logic);

end NOTGATE;

architecture RTL of NOTGATE is

begin

OUT1 <= not IN1;

end RTL;

Output:

A) AND Gate

B) OR Gate

C) NOT Gate

Result :  In this way, we have studied the simulation of basic gates AND,OR,NOT in VHDL using dataflow modeling.

Viva Questions ?

- What is logic gate?

- What are universal gates?

- Which of the two input logic gate can be used to implement an inverter circuit?

- State De-Morgan’s theorem.

PRACTICAL NO – 3

1. NAND Gate:-

Program:

Entity N is,

port(A, B: in Bit; C: out Bit);

end;

Architecture AN of N is

begin

C<= A NAND B;

end;

2. NOR Gate:-

Program:

Entity Nr is

port(A, B: in Bit; C: out Bit);

end;

Architecture ANr of Nr is

begin

C<= A NOR B;

end;

3. EXOR Gate:-

Program:

Entity Ex is,

port(A, B: in Bit; C: out Bit);

end;

Architecture AEx of Ex is

begin

C<= A XOR B;

end;

Output:

A) NAND Gate:

B) NOR Gate:

EXOR Gate:

Result:  Thus we have seen simulation and implementation of NAND, NOR and XOR gate using VHDL

Viva Questions :

1. Which are the logic gates whose all output entries are logic 1 except for one entry

there is logic 0?

2. When the output of a NOR gate is high?

3. What is a combinational circuit?

PRACTICAL NO – 4

Program:

entity mux8to1 is

port ( s : in bit_vector (2 downto 0);

d : in bit_vector (7 downto 0);

y : out bit);

end mux8to1;

architecture equation of mux8to1 is

begin

with s select

y <= d(0) when "000",

d(1) when "001",

d(2) when "010",

d(3) when "011",

d(4) when "100",

d(5) when "101",

d(6) when "110",

d(7) when others;

end equation;

Output:

Result : Thus we have seen Simulation & implementation of 8:1 Mux.

Viva questions ?

1. What is a sequential circuit?

2. Give the applications of multiplexer.

3. How many control inputs are there in 8:1 multiplexer?

4. What is a multiplexer?

PRACTICAL NO – 5

Output:

A) Half Adder:-

B) Full Adder :

Result: Thus we are designed half adder and  full adder circuit using VHDL successfully.

Viva Questions ?

1. What is sequential circuit?

2. Differentiate between combinational and sequential circuits.

3. What is a half adder?

4. What is a full adder?

5. What are the applications of adders?

PRACTICAL NO. – 6

Program:-

Entity code is

Port(b0,b1,b2,b3:in Bit; g(0),g(1),g(2),g(3):out Bit);

End;

Architecture gcode of code is

Begin

g(0)<=b0 XOR b1;

g(1)<=b1 XOR b2;

g(2)<=b2 XOR b3;

g(3)<=b3;

End;

Output:

Result: Thus, we have design & implemented binary to gray code using VHDL.

Viva Questions ?

1. What are code converters?

2. What is the necessity of code conversions?

3. What is gray code?

4. Realize the Boolean expressions for  - Binary to gray code conversion.

PRACTICAL NO – 7

PROGRAM:-

Entity dec is

Port(a,b,c: in Bit;d:out Bit_vector(0 to 7));

End;

Architecture pdec of dec is

Begin

d(0)<=(not a) and(not b) and (not c) ;

d(1)<=(not a) and(not b) and c ;

d(2)<=(not a) and  b and (not c) ;

d(3)<=(not a) and b and c ;

d(4)<= a  and (not b) and (not c) ;

d(5)<=a  and (not b) and  c ;

d(6)<= a and b and (not c) ;

d(7)<= a and b and c ;

End architecture;

Output:

Result:   Thus, we have designed and implemented 3:8 decoder successfully.

Viva Questions ?

1. What are the applications of decoder?

2. What is the difference between decoder & encoder?

3. Using 3:8 decoder and associated logic, implement a full adder?

PRACTICAL NO. – 8

Program:

Library ieee:

Use ieee std_logic_1164.all;

Entity af is

Port( idlock : In std_logic ; Q: out std_logic);

End;

Architecture aaf of af is

Begin

Process (clock)

Begin

If clock event and clock=’1’ then

Q<=D;

End if;

End process;

End aaf;

Output:

Result: Thus we have successfully designed and implemented D-flip flop.

Viva Questions ??

1. What are the applications of different Flip-Flops?

2. What are the different flip-flop?

3. What is race around in flip-flop & how to overcome it?

PRACTICAL NO. 9

LOGIC DIAGRAM:

TRUTH TABLE:

| Input Up/Down | Present State QA QB QC | Next State QA+1 Q B+1 QC+1 | A JA KA | B JB KB | C JC KC |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 0 0 | 1 1 1 | 1 X | 1 X | 1 X |
| 0 | 1 1 1 | 1 1 0 | X 0 | X 0 | X 1 |
| 0 | 1 1 0 | 1 0 1 | X 0 | X 1 | 1 X |
| 0 | 1 0 1 | 1 0 0 | X 0 | 0 X | X 1 |
| 0 | 1 0 0 | 0 1 1 | X 1 | 1 X | 1 X |
| 0 | 0 1 1 | 0 1 0 | 0 X | X 0 | X 1 |
| 0 | 0 1 0 | 0 0 1 | 0 X | X 1 | 1 X |
| 0 | 0 0 1 | 0 0 0 | 0 X | 0 X | X 1 |
| 1 | 0 0 0 | 0 0 1 | 0 X | 0 X | 1 X |
| 1 | 0 0 1 | 0 1 0 | 0 X | 1 X | X 1 |
| 1 | 0 1 0 | 0 1 1 | 0 X | X 0 | 1 X |
| 1 | 0 1 1 | 1 0 0 | 1 X | X 1 | X 1 |
| 1 | 1 0 0 | 1 0 1 | X 0 | 0 X | 1 X |
| 1 | 1 0 1 | 1 1 0 | X 0 | 1 X | X 1 |
| 1 | 1 1 0 | 1 1 1 | X 0 | X 0 | 1 X |
| 1 | 1 1 1 | 0 0 0 | X 1 | X 1 | X 1 |

Output:

Procedure:

- Connections are given as per circuit diagram.

- Logical inputs are given as per circuit diagram.

- Observe the output and verify the truth table.

Result: Thus we have studied to design and implement 3 bit synchronous up/down counter.

Viva Questions???

1. What are synchronous counters?

2. What are the advantages of synchronous counters?

3. What is an excitation table?